JOMO KENYATTA UNIVERSITY

OF

AGRICULTURE AND TECHNOLOGY

BSc. MARINE ENGINEERING

PROJECT REPORT

DESIGN, FABRICATION AND TESTING OF A LOCALLY REMOTE OPERATED VEHICLE FOR SHIP HULL INSPECTION

| PROJECT CODE: | MR-PRO-24-04 |  |
| --- | --- | --- |
| STUDENTS: | DENNIS SAMMY | ENM 241-0117/2019 |
|  | PEGGYOLANDA NYAMBURA | ENM 241-0118/2019 |
| SUPERVISOR: | MR. ELIJAH MUNYAO |  |

This project report has been submitted in partial fulfillment of the requirements for the award of degree of Bachelor of Science Marine Engineering

DECLARATION

We declare that this project is our original work. To the best of our knowledge, we also affirm that this project has not been presented to this university or any other university for examination, or for any other purpose.

| Name: | DENNIS SAMMY | ENM 241-0117/2019 |
| --- | --- | --- |
| Signature: | ____________________ | Date __________________ |

| Name: | PEGGYOLANDA NYAMBURA | ENM 241-0118/2019 |
| --- | --- | --- |
| Signature: | ____________________ | Date __________________ |

CERTIFICATION

I certify that the above-mentioned students carried out the work detailed in this project under my supervision.

| Supervisor’s Name: | MR. ELIJAH MUNYAO |  |
| --- | --- | --- |
| Signature: | ____________________ | Date __________________ |

# Abstract

Hull inspection is an essential process for any sea going vessel to ensure safety of navigation. Over the years, hull inspection has been achieved through dry docking as well as deploying inspectors to carry out underwater inspections. However, this has had limitations due to the inadequate human capabilities underwater as well as the associated safety risks. Remotely operated vehicles eliminate the risks that divers are subjected to during hull inspections and minimises expenditure on the entire hull inspection process in dry docks. The use of an ROV as an underwater inspection tool provides precise and efficient underwater inspections at reduced risks, associated costs, and docking time periods. The current work involved the design, fabrication, and testing of a remotely operated vehicle. It is worth noting that most vessels have greater draughts more than 7 m. Consequently, the design is based on a draught average of 10m which is greater than the draughts of majority small marine vessels such as Aylah 1 [1].

Table of Contents

Abstract	4

List of Figures	7

List of Tables	8

List of Abbreviation	9

1. Introduction	10

1.1 Background	10

1.2 Problem statement	11

1.3 Objectives	11

1.3.1 Specific objectives	11

1.4 Scope	11

2. Literature review	12

2.1 Work Class ROV	13

2.2 Observation Class ROV	14

2.3 Micro ROV	14

2.4 Work Class ROV	14

2.5 Special Use ROVs	14

3.0 Methodology	16

3.1 Conceptual Design	16

3.1.1 Structural design	16

3.1.2 Propulsion mechanism Design	20

3.1.3 Electrical and control design	24

3.2 Fabrication and assembly	29

3.3 Testing	30

4. Results and Discussion	31

4.1 Research and selection of potential materials for key components.	31

4.2 Analysis of proposed materials	31

4.3 Determining the weight and displacement of the ROV.	43

4.4 Calculating the buoyant force, hydrodynamic drag force, power of the motor, and the thrust Force.	45

4.4.1 ROV Upthrust Force	45

4.4.2 ROV buoyancy	45

4.4.3 ROV hydrodynamic drag	45

4.5 The electrical design	48

5. Conclusion	50

6. Budget	51

7. Time plan	52

References	53

Appendix	55

# List of Figures

Figure 1: Rebikoff-niggeler Foundation First ROV	13

Figure 2: ROV explores the ocean depth.	14

Figure 3: Arduino nano	26

Figure 4: Raspberry pi 3	26

Figure 5: Pressure sensors	26

Figure 6: Electronic System Architecture	28

Figure 7: L293D IC Chip	29

Figure 8: An ethernet cable used for transmission of data.	29

Figure 9: Frame model	32

Figure 10: Stress comparison at a depth of 10 m	35

Figure 11: Deformation comparison at a depth of 10 m	35

Figure 12: Deformation comparison of each material at different depths	41

Figure 13: Stress comparison of each material at different depths	41

Figure 14: Modeled acrylic sealing compartment	42

Figure 15: Schematic circuit Main Board	47

Figure 16: Schematic circuit Motor and motor drivers	48

Figure 17: Schematic circuit sensors and modules	48

# List of Tables

Table 1: Pros and cons of proposed material	20

Table 2: Technical characteristics of extisting thruster model	23

Table 3: Principal Dimension of the ROV	31

Table 4: Pressures at different depths	32

Table 5: Von Mises Stress and Deformation Using Steel	33

Table 6: Von Mises Stress and Deformation Using aluminum	33

Table 7: Von Mises Stress and Deformation using PVC.	33

Table 8: Von Mises Stress and Deformation Using Acrylic	34

Table 9: Von Mises Stress and Deformation of different materials at 10m	34

Table 10: Stress and Deformation of Steel on Depth Variation	36

Table 11: Stress and Deformation of Steel on Depth Variation - values	36

Table 12: Stress and Deformation of Aluminum on Depth Variation	37

Table 13: Stress and Deformation of Aluminum on Depth Variation - values	37

Table 14: Stress and Deformation of PVC on depth variation	38

Table 15: Stress and Deformation of PVC on depth variation - values	38

Table 16: Stress and Deformation of Acrylic on depth variation	39

Table 17: Stress and Deformation of Acrylic on depth variation - values	39

Table 18:Properties of materials	40

Table 19: Allowable stress for the different materials	40

Table 20: Stress and Deformation of Acrylic sealing compartment at 100m depth.	42

Table 21: Stress and Deformation of Acrylic on depth variation - values	42

Table 22: Total Mass of the ROV	43

Table 23: Budget	56

Table 24: Time Plan	57

# List of Abbreviation

| Remotely operated vehicle | ROV |
| --- | --- |
| Polyvinyl chloride | PVC |
| Electronic Speed Controller | ESC |
| Inertial Management Unit | IMU |
| Pulse Width Modulation | PWM |
| Graphic User Interface | GUI |
| General Purpose Input/Output | GPIO |

# 1. Introduction

## Background

In the shipping industry, ships are required to be inspected over a given period as safety measures [2]. Currently, there are significant challenges associated with conducting hull inspections. These include the high cost, a time-consuming process involved in inspection of hulls in dry docks. In cases where underwater hull inspections are adopted, the inspectors are exposed to a myriad of safety risks as well as the inability to access some areas of the submerged hull. The hull is exposed to underwater processes such as biofouling and corrosion, making its inspection vital and therefore it should be conducted regularly, which is not feasible with the currently available method. Both Biofouling and corrosion have a great negative impact on the performance, condition and lifespan of the ship. Biofouling has also been identified as a contributor to increased fuel consumption for sea going vessels, consequently contributing to increased emission ozone depleting gasses - the greenhouse gasses.

Currently, the existing hull inspection methods include dry docking and use of underwater hull inspectors [3].  In dry docking, a ship is unsubmerged for a comprehensive inspection to be conducted on the hull, the rudder, propeller, and stabilizers. Dry docking has proven to be expensive and time consuming considering the required labour, resources and planning. The docking facilities are also not readily available resulting to delays in inspection. Another alternative involves utilizing underwater inspectors to evaluate the status of the hull. Even though this reduces the amount of resources and cost expended in the inspection, it has proven to be a difficult and risky endeavour. It is life threatening as it exposes the inspectors to hydrostatic pressures, low visibilities, marine predators, rough and harsh sea conditions [4]. Due to these limitations, there is a need to develop alternative reliable and safe methods to conduct hull inspection. One of the feasible approaches is to use a Remotely operated vehicle (ROV).

A Remotely operated vehicle (ROV) is a manoeuvrable underwater machine that can be used to explore and inspect ocean depths operated by personnel at the water surface. ROVs are versatile tools that find applications in oil and gas companies, marine construction works, military and law enforcements applications. They are also used in underwater research as well as in shipping industry, bringing forth numerous benefits in these areas of application [5]. ROVs allow the accessibility of inaccessible underwater areas. They also guarantee improved safety and operate with low maintenance. They have been used to recover torpedoes, bombs, and mines from the 1960s by the navy as well as to carry out missions that are very dangerous to humans such as mine breaking and mineral harvesting [6]. ROVs have also been used by the leading coating provider, Hempel, to perform remote hull inspections [7].  It is based on the above discussion that this project implements a locally made ROV for hull inspections.

## Problem statement

Inspection of ship hulls and offshore structures is a major challenge in the maritime sector. Conventional hull inspection methods such as docking, and use of underwater inspectors have proven to be costly and risky. An alternative way to overcome these challenges is to create a device which will fulfill the underwater duties. Remotely Operated Vehicles (ROV’s) have become vital tools in industries that are related to underwater. Deployment of ROVs has been proven to reduce the inspection cost from the previously existing methods but still costly for the local people. The existing ROVs are relatively expensive, and this is a prohibitive barrier to the entry into application, preventing widespread adoption of ROV for personal, research, and conservation applications. The current work therefore seeks to address the challenge by coming up with a low cost locally available Remotely Operated Vehicle without compromising its functionality.

## Objectives

The main objective of the current work is to design, fabricate and test a local Remotely Operated Vehicle (ROV) for hull inspection.

### 1.3.1Specific objectives

- To select and analyse materials for the ROV.

- To design and test the structural integrity of the ROV frame.

- To design, fabricate and test the control system of a Remotely operated vehicle (ROV).

## Scope

The scope of this project is to design, fabricate and test the control system of the Remotely Operated Vehicle. The ROV should maintain structural integrity as well as display manoeuvrable controls along the main axes of motion and around an underwater obstacle.

# 2. Literature review

Over time, humans have creatively built tools to improve lives and the ROV is no different. The human desire to reach places underwater that are too dangerous or too deep for a diver to venture into has seen the development of ROVs. ROVs are unoccupied machines designed for specific underwater missions. Typical ROVs are controlled from a vessel, land, or a floating platform.  The first tethered ROV referred to as POODLE was developed by DimitriRebikoff in 1953-1954. The United States navy later advanced the ROV technology to recover bombs, mines, and torpedoes in the 1960s that had been lost during tests. ROVs gained popularity when the Cable Controlled Underwater Recovery Vehicle systems were used by the US Navy to recover an atomic bomb lost off Palomares Spain and then saved pilots of a sunken submersible off Cork in Ireland with only minutes of air remaining [8].

Figure 1: Rebikoff-niggeler Foundation First ROV

The ROVs were also used on rescue missions that exceeded the reach of human divers such as mine hunting and mine breaking. Since the 1980s, the offshore oil industry has successfully used ROVs to assist in offshore development, especially as waters became too deep for human divers. ROVs have become famous for discovering historic shipwrecks, including the famous RMS Titanic in 1985 and the Bismarck [9]. The first ROVs were initially referred to as Cable-Controlled Underwater Recovery Vehicles (CURV) since their primary function was to lead search and rescue operations in deep and dangerous waters.

ROVs consist of video cameras that transmit real time surveillance to scientists aboard vessels and this should be with low latency, sonar systems and a buoyancy pack that allow the vehicle to remain light and easy to manoeuvre when in the water. Thrusters are used to manoeuvre the vehicle, lights to provide illumination for the camera underwater, a frame that provides a structure to attach the ROV components, and a tether which carries electrical power and signals to the surface. ROVs use external sensors that are mounted on the vehicle to measure properties or parameters such as temperatures and depth. ROVs may be built with end effector such manipulators for collecting biological and geological samples.

Some ROVs are built with two bodies such as the NOAA Ocean Exploration’ vehicles [10]. The advantage of a two-body system is that the hovering ROV acts as an extra light source and camera giving pilots, scientists, and viewers an expanded view of the ocean. However, this has resulted in difficulties in control and transportation of single body counterparts. ROVs range in size, likewise the prices also vary a great vast too depending on the size and application of the ROV. They normally start at thousands of dollars and can go up to millions of dollars. The figure below shows an ROV using LED lights to explore the ocean floor.

Figure 2: ROV explores the ocean depth.

Among the prominent ROVs is ROV Ventana. ROV Ventana was constructed by International Submarine Engineering in 1987 and launched by MBARI’s research vessel and has been in operation since 1988. It runs on hydraulic power, exploring waters at a depth of 1800 meters [11].

Underwater ROVs can be classified into various classes based on their weight, power, abilities and sizes. The common classes include the work class ROV, light work class, observation class ROV, and the micro ROV.

### 2.1 Work Class ROV

This class of ROV is used for ocean floor exploration and inspections at depths that divers are often unable to reach. Oil and gas exploration and production activities in the marine environment are increasing. In addition, the amount of global hydrocarbon reserves removed from below the seafloor is set to increase, with exploration heading into deeper, more remote waters, many of which are yet to be fully explored [12]. Work Class ROVs act as a safe alternative to divers and are often used in offshore energy projects and deep archaeological explorations.

### 2.2 Observation Class ROV

This class of ROV is small and is used to explore lakes, rivers, and coastal waters. They are often used to test water safety prior to a diver entering the water during missions and conduction inspections. They are equipped with sonar and custom sensors that make them versatile underwater vehicles.

### 2.3 Micro ROV

The ROVs in this class are the smallest and are often used to inspect hard to reach areas at shallow depths such as pipe systems and submerged infrastructure. They explore miniscule cavities and pipeline cracks which are physically impossible for divers to achieve.

### 2.4 Work Class ROV

This class of ROV is used for ocean floor exploration and inspections at depths that divers are often unable to reach. They act as a safe alternative to divers and are often used in offshore energy projects and deep archaeological investigations.

### 2.5 Special Use ROVs

They describe tethered underwater vehicles designed for specific purposes. An example of a special-use vehicle is a cable burial ROV system designed to plow the sea floor to bury telecommunications cables.

ROVs have great benefits that enable them to be a prospective candidate in providing a solution to underwater exploration, inspections, and adventures. They have been able to have quick deployment due to the compact design and easy-to-use inspection technology. This is highly beneficial in emergency situations and in areas that are narrow or difficult to access. ROVs are robust and rugged in design thus can withstand harsh water environments and require minimal maintenance. Depending on sea conditions and the type of operations, divers can only remain underwater for a limited amount of time; 30 min to an hour at a time. ROVs have overcome the challenge as they are designed to operate for long hours and in difficult conditions. One of the most remarkable benefits is the photo and video recording capabilities. ROVs can provide high resolution footage even in dark and murky waters. ROVs can be deployed to record and later be reviewed for documentation and results. In Addition, ROVs provide safe alternatives for exploration and cost effectiveness comparable to traditional methods [13].

Several industries have been able to benefit from the use of underwater ROVs technology. These industries include Military, Aquaculture, Municipal Infrastructure, Commercial and Salvage Diving, Ocean Science, Oil and energy, Shipping, Underwater Discovery.

# 3.0 Methodology

The current work is carried out systematically in three main steps: conceptual design, fabrication, and testing.

## 3.1 Conceptual Design

Conceptual design comprises the design of structural, electrical and control systems. The design aims to reduce the cost of the ROV by considering locally available materials and technologies without compromising the functionality of the ROV.

### Structural design

Structural design is an essential key part of the project as it assists in coming up with a sturdy and robust structure for the ROV. This design entailed:

- Determining the main dimensions; length, width, height.

- Determining weight of the ROV.

This provides the basis to create a 3D model that is essential in the successive sections of the design. A key part of the structural design is the selection of materials that are used to form the structure to withstand the high hydrostatic and hydrodynamic pressures that the ROV will be subjected to during its operation.

#### Design constraints

The major constraints that affect the design process of the Remotely Operated Vehicle include:

- Environmental constraints

The vehicle is to operate in sea waters, thus the parameters that will be experienced are depth, density and salinity.

- Density and salinity
The density of water is affected by temperature and salinity. Water is at its highest density at 4 degrees Celsius and the density decreases with an increase in saline concentration.

- Working depth
The working depth defines the mechanical strength of all structural components. The current work suggests an operational depth of 10 m. This depth establishes a secure margin for the collapse resistance of the structural elements. Thus, the structural calculations are done at the design depth to guarantee the integrity of the ROV.

#### Hydrostatic and hydrodynamic considerations

A list of hydrostatic and hydrodynamic considerations is addressed by Allmendinger [14]. Such considerations include loads of interest, static equilibrium conditions, static and dynamic stability conditions and drag and thrust considerations.

- Load of interest

The loads that affect the ROV include weights, displacement force due to change in pressure, and drag and lift force as a result from the motion of the ROV in water,

- Static equilibrium

The buoyancy of the ROV is the difference between the displacement and its weight. The buoyancy is positive when the displacement is greater than weight and it is negative when the displacement is less than the weight. It is neutral when the displacement and the weight are equal. It is desired that the buoyancy be positive or neutral. This is achieved when the difference between the displacement and vehicle's weight be small and positive. [15]

- Static and dynamic stability conditions

Static stability occurs when the heeling moment increases very slowly from zero to a maximum value while dynamic stability is when the heeling moment changes rapidly. Static stability is the most important measure of heeling behavior of a ship. A vessel submerged in water will always seek to assume a position in which all forces and moments acting upon it cancel each other out, which means that it is in equilibrium when its weight and buoyancy are equal but of opposite directions [16].

- Drag and thrust.

The thrust power must be able to overcome drag at an operational speed, and during acceleration intervals.

#### Design Considerations and requirements

The main parts of the ROV include the frame, the sealing compartment, thrusters, camera, lighting equipment, control mechanism and buoyancy mechanism. The frame plays an important role of support and protection of the equipment [17]. It is thus necessary the following factors should be considered in the design of the ROV:

•	Withstand pressure at the designed depth.

•	Cost effective.

•	Lightweight to ensure a neutral or negative buoyancy

•	Water corrosion resistance.

•	Material Machinability.

•	Robust and Sturdy.

#### Pressure Calculation

There have been previous attempts to build the ROVs as documented in [18]. With the desired operational depth of 10m, pressure can be calculated to determine whether the selected structural material for the frame is suitable.

|  | 1 |
| --- | --- |

Where P is the pressure, p is the density of water in Kg/m3, g is the gravity and h is the depth in meters. Taking p as density of sea water to be 1025 kg/m3, pressure is calculated using equation [1]. Where P is hydrostatic pressure in MPa, h is 10m, g is 9.81m/s2 and density of seawater is 1025 kg/m3

10×9.81×1025=100552.5 Pa

#### Selection of materials

The frame of the ROV, the sealing compartments and the end caps are the three key components of focus in material selection.

- The Frame

The frame is the structural architecture that will hold the components that will drive and execute the functions of the ROV. The materials proposed include aluminium, Stainless Steel, unplasticized PVC (Polyvinyl Chloride), and Acrylic- Plexiglass [19]. The pros and cons of each material are shown in table [1] below.

Table 1: Pros and cons of proposed material

| Material | Pros | Cons |
| --- | --- | --- |
| PVC | Light in weight Excellent corrosion resistance Cost effective. Readily available. Durable | Susceptible to UV degradation Lower strength in comparison to metals |
| Aluminium | Light in weight Easy to machine and fabricate. Good corrosion resistance | Requires proper anodizing |
| Acrylic Plexiglass | Corrosion resistant Light in weight | Expensive Complex manufacturing processes |
| Stainless steel | High strength and durable Good corrosion resistance | Heavy Expensive |

- The Sealing compartment

The sealing compartment is used to protect and house the electronic components of the ROV. The material used to make the sealing compartment of electronic components in Remotely Operated Vehicles (ROVs) is critical for ensuring the integrity and functionality of the electronics in underwater environments. Acrylic plexiglass can withstand the pressure at the designed depth. In addition, Acrylic offers excellent light transmittance and UV resistance. This is beneficial for visibility and protection in marine environments. [20]

- The End Caps

End caps are used to enclose the openings of the cylindrical sealing compartments. They are to be machined using Teflon, which is commonly used on bearings and low-friction bushings due to its: low friction, thermal resistance, and corrosion resistance [21]. The design consists of 4 O-rings on the end caps as well as a wide flange nitrile gasket on the face of the dome and the fixed dome connecting the aluminium mount.  A nylon ring is machined so that it presses the transparent dome against the front cap once sealed with silicone so that no water enters.

#### Analysis of proposed materials

Selection of the best material for the frame entails subjecting the proposed materials to a stress and deformation analysis test under load. The modelled design is subjected through a series of pressures at different depths of 10m, 50m, and 100m and the results are reviewed. Fusion360 is used to analyse the structure for stress and deformation. The mechanical properties, thickness, and allowable values of each material are then summarized.

### 3.1.2 Propulsion mechanism Design

The propulsion system allows the ROV to utilize underwater thrusters in a specific configuration to grant an operational range of motion converting input energy into useful thrust. The propulsion system consists of motors and propellers - a combination called a thruster. The size of the propeller is described by a pitch and diameter number. Rake is the degree that the blades of a propeller slant forward or backwards in relation to the hub which is the centre section of the propeller. The aim of the design of a ROV propulsion system is to have high thrust to physical size and power input ratios [22].

A propeller is selected with consideration for performance, speed, weight and the overall size of the ROV. Ducted propellers move more liquid, and they increase the efficiency depending on the flow rate [23]. The selected propeller features three blades, a small diameter, a high rake angle and nylon polymer construction. Three bladed propellers are chosen because of the size of the ROV and the overall cost of the propeller. The smaller diameter propeller helps in avoiding collision with any of the frame components and to provide maximum thrust capabilities from the thrusters selected. A high rake angle allows for faster acceleration at lower rotations per minute thus allowing the ROV to respond rapidly to changes and reposition [24].  Due to weight and cost restrictions, nylon plastic propellers are selected to ensure the best performance to cost ratio.

#### Thrusters

As the propeller rotates in ROV, high-pressure and low-pressure areas are formed between the blades. The thrust needed for the movement of the ROV is formed as the fluid moves from the high-pressure area to the low-pressure area. The design of the thrusters ought to allow for components to be replaced in future in case of a component upgrade or a part failure. The individual thrusters are to have identical propellers to allow for each propeller to have uniform performance during operation [25].  This allows the operator to have more uniform control of the ROV despite external water conditions. As the propeller moves, there are losses at the tip of the propeller blades. To minimize the losses, a duct is designed around the propeller. The duct is designed with an aerodynamic structure to increase the efficiency of the design.

Brushless motor thrusters are chosen for the ROV since they are efficient and durable. The diameter of the propeller is chosen to be bigger than the motor diameter for enhanced thrust efficiency, reduced cavitation, lower power consumption, improved control and stability, optimized flow characteristics and hydrodynamic performance. The pitch of the blades depends on the diameter and the rotational speed of the motor in RPMs. The width of the blade determines the amount of water it pushes, thus light blades are used for higher speed applications [26]. They are designed to produce forward and reverse thrust. Each thruster is controlled using an electronic speed controller (ESC).

#### Thruster configuration

The ROV is designed with five thrusters. The thrusters mounted involve two thrusters moving in the horizontal motion along with three vertical thrusters to dive. Two thrusters are placed on the back of the ROV equidistant from the centre of momentum in the X-direction to minimize rotation. They provide translational motion in the X-direction. The remaining three thrusters are placed on the top of the ROV which provides the motion in the Z-direction. They are placed along the X-axis for the ROV to obtain pitch rotation by only firing the front thruster for pitch down and firing only the back thruster for pitch up. This configuration allows to actively control the system in six degrees of freedom. The basis of this ROV system consists of 5 Blue Robotics T100 thrusters, which have the following technical characteristics.

Table 2: Technical characteristics of existing thruster model

| Property | Description |
| --- | --- |
| Thrust | 14.7 N forward and 11.77N reverse |
| Dimensions | 72 by 92 mm |
| Mass | 0.075kg |
| Electric Power | Up to 150W when powered from 12V DC |
| Supply voltage | 12-24V |

#### Analysis of upthrust, buoyancy, drag force, power of the motor and the thrust Force.

Calculation of Upthrust Force:

Upthrust is the force exerted by a fluid on an object in an upward direction. It is equivalent to the weight of the fluid displaced by the submerged part of the body. Upthrust is calculated using equation [2] below such that m is the mass of the body and g is the acceleration due to gravity.

|  | 2 |
| --- | --- |

Calculation of buoyant force.

The buoyant fore is the upward force exerted by the water on the ROV, which counteracts its weight. It is calculated using equation [3] below

|  | 3 |
| --- | --- |

where p is the pressure, g is the gravity and V is the volume of the ROV.

The ballast weights and syntactic foam are either added or subtracted to fine tune the ROV’s buoyancy and achieve a neutral buoyancy.

Calculation of Drag force.

The thrust required to achieve a design speed of 2m/s depends on the hydrodynamic drag force acting on the ROV. The drag force on an ROV moving through water can be calculated using the drag equation [4].

|  | 4 |
| --- | --- |

Where:

ρ is the density of sea water= 1025kg/m3 and gravitational acceleration = 9.8m/s2

v is the desired velocity (2 m/s)

Cd​ is the drag coefficient

A is the characteristic cross-sectional area of the ROV

The total drag of the ROV system is the vehicle drag and the tether drag. The coefficient of drag for tether cables used in Remotely Operated Vehicle (ROV) applications vary depending on the cable's surface characteristics, diameter, and flow conditions. The tether drag coefficient in a turbulent flow regime is approximated to be 1.2 [27].

|  | 5 |
| --- | --- |

The total hydrodynamic drag force on the ROV system is thus the summation of the tether drag and the vehicle drag, and it is given by equation [6] below.

|  | 6 |
| --- | --- |

Power of motor analysis

To calculate the power of a motor needed to achieve a speed of 2m/s and overcome a drag force of the fundamental relationship between power, force and velocity is applied in equation [7] below

|  | 7 |
| --- | --- |

The thrust force, FT, to achieve a desired speed is given by equation [8] below:

|  | 8 |
| --- | --- |

Where P is Power in watts, F is force in newtons, V is velocity in m/s,

### 3.1.3 Electrical and control design

The electronic and control circuitry design of a Remotely Operated Vehicle (ROV) enables precise operation, communication, and functionality of the ROV in underwater environments. This design includes the integration of microcontrollers, sensors, and communication modules to process operator commands and relay real-time data. Control circuitry governs the operation of thrusters ensuring smooth and accurate movements. The electronic equipment is securely sealed inside the electronic enclosure.

Electrical devices

The electrical devices of the Remotely Operated Vehicle (ROV) include key components that enable its functionality, control, and communication in underwater environments. These devices typically consist of thrusters for propulsion, cameras for visual feedback, and sensors to measure parameters such as depth, temperature, and water pressure. Raspberry pi 4 is used to process commands and control the ROV's

Power system

The power system describes how the ROV components are powered. This project used batteries as the power source. The batteries are to be reusable hence rechargeable li-on batteries are used. Due to the presence of motors that are used to power the thrusters more power is drawn from the batteries. A proposed solution is to allocate a second power source coupled with relays and sensors to provide power to the motors when needed.

Control system

The control system manages the input from the operator controller at the surface and converts it into control motion of the ROV underwater. Measurable parameters such as voltage, battery life, depth, and position of the ROV are recorded by sensors and displayed to the operator.

Microprocessor and microcontroller

The microprocessor and the microcontroller are responsible for executing parallel tasks such as motor control, measurement and recording of different sensors of the system, battery voltage monitoring for energy consumption and communication management with remote control. The Raspberry PI 4 is the selected microprocessor. It is a compact, affordable, powerful and versatile single board computer (SBC) that served for the control system of the ROV. A camera module connects to the board of the Raspberry pi 4 via the Camera Serial Interface (CSI) port using a flexible ribbon cable. The Camera Serial Interface port is specifically designed for high-speed data transmission between the camera and the processor.  The image sensor captures light and converts it into electrical signal which is digitized and processed by the Graphic Processor Unit. The processed image data is then transmitted from the camera module to the raspberry pi 4 over the CSI. The processed image is then displayed on the screen.

and Arduino nano will be used as microcontroller. Figures [3] and [4] below show the microprocessor and the microcontroller.

| Figure 3: Arduino nano | Figure 4: Raspberry pi 3 |
| --- | --- |

Sensors

Pressure and depth information is essential while the ROV is in operation. Pressure sensors, integrated with the code, can determine the information on depth. Figure [5] below illustrates a pressure sensor.

Figure 5: Pressure sensors

Electronic speed controllers (ESC) regulate the speed and direction of motors based on the input signals. Upon signal reception from the raspberry pi 4, the microcontroller inside the ESC interprets the input signal and determines how much power to send to the motor thus varying the width of the (Pulse Width Modulation) PWM. The Metal Oxide Semiconductor Field Effect Transistor (MOSFET) rapidly switches the DC power received by the ESC to the motor’s 3 Phase AC signal in a sequence. They include a pair of relays for each motor. The sensors required for the inspection are added as payload and integrated with the circuitry. Ethernet cable used as the tether for transmission of control signals, the camera feed, and the sensor data from the ROV.

Various sensors specific to the application are equipped to carry out detailed underwater monitoring. These sensors also serve as a health monitor to prevent its navigation beyond the operating range.

The brushless motor has a rotor which is the permanent magnet and the stator which has copper coils. When the current is delivered to the coil of the stator, the coil becomes an electromagnet and develops polarities which repel the rotor magnet causing it spin. The 3 Phase AC current delivered by the ESC to the motor constantly changes the polarity of the electromagnet and this keeps the rotor spinning. Hall sensors incorporated withing the motor precisely track the rotor’s position.

The ROV is equipped with a temperature sensor with a usable temperature to monitor the temperature underwater for any alarming temperature variations. The pressure sensor is used to provide real-time pressure data that helps in determining the depth of the ROV. It also serves as an external aiding sensor for position tracking by providing depth. The data from these sensors are serially communicated to the microcontroller which is connected to the processor. They are powered by a 5V converter. These sensors are placed along the inner walls of the ROV frame and wired into the enclosure via a slotted acrylic flange.

The Inertial Measurement Unit (IMU) contains a gyroscope, accelerometer, and magnetometer for providing compensated incremental angle, velocity data for navigation and angular rates, linear accelerations, and magnetic fields for control through a digital serial interface bus. The IMU measures angular rates(rad/sec), linear acceleration(m/sec2), and magnetic fields(milli-gauss) in a body mounted strap down configuration along the three axes which are fed to a sensor fusion algorithm to obtain the attitude and heading of the ROV with respect to the Earth Centred Inertial frame. The navigation data involving the incremental angles and velocities along the three axes are primarily used for positioning the ROV while the control data is used in the determination of the orientation in Euler angles (roll, pitch, and yaw) and control of the vehicle. Since stand-alone gyroscope has no frame of reference in contrast to accelerometer with earth’s acceleration due to gravity as the reference and magnetometer with the earth’s magnetic north as a frame of reference, all the three sensors are fused to obtain the complete orientation of the sensor with respect to a navigation frame. The IMU is powered with a 5V converter. It is mounted vertically on the flat tray at the centre of gravity of the vehicle such that the x-axis of the IMU coincides with the direction of the vehicle’s heading. Further calibration of the IMU after placing it at the COG is done to reduce the bias and obtain precise values.

Electrical system architecture

Figure 6 shows the block diagram of the hardware that consists of two parts: the ROV vehicle system and the operator system. The two subsystems are communicated through an Ethernet Cable so that the user can control the ROV from a personal computer, laptop.  The ROV hardware involves a Raspberry Pi 4 that is responsible for executing parallel tasks, such as:

- Acquisition of video by means of a digital camera in coordination with an Arduino Nano microcontroller.

- Measurement and recording of the different variables associated with the sensors of the system.

- Motors control.

- Battery voltage monitoring for energy consumption and internal temperature in coordination with an Arduino Nano microcontroller.

- Communication management with the remote control.

Power sources for the Raspberry Pi 4, an Arduino Nano microcontroller, and digital sensors, and the other one is a bank of five batteries of to power five thrusters’ motors. The navigation control consists of buttons series interconnected by an Ethernet network cable to onboard computer, from which the user controls the ROV through a Graphical User Interface (GUI).

Figure 6: Electronic System Architecture

The thrusters are controlled by a L293D motor drivers, with the speed control having as an input a pulse width modulation (PWM) signal generated by the raspberry pi 4 microcontroller. In this project, python scripts are used to run on a Raspberry Pi 4 to set General Purpose Input/Output (GPIO) outputs to an L293D motor controller IC and run thruster in either direction at any speed. The Raspberry Pi 4 has 40 open General-Purpose Input/Output (GPIO) pins which allow the user to interface with external devices such as motors, and sensors. The pins include power pins, ground pins, special function pins and digital Input/Output pins. The GPIO can either send electrical signals to drive hardware or receive electrical signals and read sensor data. They are thus used as outputs, to send signals to the L293D IC Chip, which is a chip used to control thrusters. The motor is controlled using a python module, Pulse Width Modulation (PWM) controls the amount of time the voltage is on by flipping between high and low for a set amount of time. The amount of time the voltage is high is the ‘duty' or 'duty cycle', and the percentage is the power the motor runs on.

The L293D motor IC uses two pins that are inputs to sense the desired direction of the output, and the Enable pin used to sense On/Off. When the motor is desired to spin forward, input 1 is set to true or high and the second input is set to false or low. When the motor is desired to spin backwards, input 1 is set to false or low while input 2 is set to true or high.

Figure 7 below illustrates a L293D IC Chip

Figure 7: L293D IC Chip

Tether management system

The ROV use tether, high-bandwidth communication cable for communication with the operator. The fibre optic tether enables the transfer of high-definition videos. The figure below depicts the ethernet cable used for data transmission.

Figure 8: An ethernet cable used for transmission of data.

Code implementation

Python has extensive libraries and cross-platform compatibility. It is thus the base of modern programming of raspberry pi 4 microprocessor.

## Fabrication and assembly

Implementing the ROV Electrical and Control design followed a series of processes that involved acquisition of electrical components and integration of the software and hardware, and the assembly of the ROV. A clear acrylic dome will be installed to enable the survey without exposing the camera to water and eliminating the need for special waterproofing. LEDs are installed for illumination. To prevent water leakage into the ROV and keep the electronic components safe, O-rings will be used to seal off the electrical compartment.

## Testing

Testing - a critical phase in the design of a ROV – ensured the functionality of the designed control and electrical system of the ROV to achieve manoeuvrability in the main axes of motion and around an obstacle.

# Results and Discussion

## Research and selection of potential materials for key components.

The ROV is to conduct hull inspection on a vessel and thus the material selected should withstand the pressure experienced at the drought of the vessel. Settling on an average depth of 10m, which is greater than the draughts of majority small marine vessels such as Aylah 1 and Falcao, the pressure is calculated.

Calculation of Pressure

Pressure is calculated using equation [1]

Where P is hydrostatic pressure in MPa, h is 10m, g is 9.81m/s2 and density of seawater is 1025 kg/m3

| 10×9.81×1025=100552.5 Pa | 9 |
| --- | --- |

## Analysis of proposed materials

The model development study was carried out on the existing ROV of the dimensions given in table 3 and it was modelled to the design plan. To maximize the resulting velocity from a given propulsive force, the ROV ought to displace the least amount of water possible along its movement [28]. Thus, there should be an absolute minimum of surface area that obstructs water flow through the ROV yet retain structural integrity to withstand the propulsive forces without any deformation. A smaller frame that still has minimal solid obstructions is ideal, yet still places the thrusters in a manner to ensure manoeuvrability in the respective degree of freedom.

Table 3: Principal Dimension of the ROV

| No. | Dimension | Overall Size (mm) |
| --- | --- | --- |
| 1. | Length | 550 |
| 2. | Width | 550 |
| 3. | Height | 211.13 |
| 4. | Thickness | 5 |

The design considerations for the frame are:

• Simplicity and rigidity

• Symmetry in thrusters’ distribution to minimize undesired reaction moments.

• The vehicle should be nearly neutrally buoyant, and the centre of buoyancy should be placed above the centre of gravity to generate righting moment.

Figure 9: Frame model

The analysis was taken through a series of 3 depth variations i.e. 10m, 50m, and 100m. Table 4 below shows the hydrostatic load used.

Table 4: Pressures at different depths

| No | Depth | Load (Pa) |
| --- | --- | --- |
| 1 | 10 | 101,043 |
| 2 | 50 | 505,215 |
| 3 | 100 | 1,010,430 |

The first analysis was carried out on each material to establish variation effect to the model with the load from 10m water depth and the results were tabulated in table 5, 6, 7, 8 and 9 below. Figures 9 and 10 below provide stress and deformation comparisons for the materials at a depth of 10 m.

Table 5: Von Mises Stress and Deformation Using Steel

Table 6: Von Mises Stress and Deformation Using aluminium

Table 7: Von Mises Stress and Deformation using PVC.

Table 8: Von Mises Stress and Deformation Using Acrylic

Table 9: Von Mises Stress and Deformation of different materials at 10m

| No | Material | Deformation(mm) | Deformation (sigmoid function) | Stress (MPa) | Stress (sigmoid function) |
| --- | --- | --- | --- | --- | --- |
| 1 | Steel | 0.003 | 0.5007499994 | 5.894 | 0.9972516434 |
| 2 | Aluminium | 0.008 | 0.5019999893 | 5.809 | 0.9970085466 |
| 3 | PVC | 0.758 | 0.6809193547 | 5.577 | 0.9962303637 |
| 4 | Acrylic | 0.208 | 0.5518133302 | 5.741 | 0.996798726 |

Figure 10: Stress comparison at a depth of 10 m

Figure 11: Deformation comparison at a depth of 10 m

The stress and deformation results of each material at depths of 10m, 50m and 100m are shown in tables 10,11,12,13, 14, 15, 16 and 17 below. Table 18 below depicts the properties of each material while table 19 below shows the allowable stresses of each material. Figure 12 and 13 below depicts deformation and stress comparison of the materials respectively at depths of 10m, 50, and 100m.

Table 10: Stress and Deformation of Steel on Depth Variation

| Depth (m) |  |  |
| --- | --- | --- |
| 10 |  |  |
| 50 |  |  |
| 100 |  |  |

Table 11: Stress and Deformation of Steel on Depth Variation - values

| STAINLESS STEEL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Depth(m) | Load (Mpa) | Deformation(mm) | Deformation(sigmoid) | Stress (Mpa) | Stress(sigmoid) |
| 1 | 10 | 0.101043 | 3.00E-03 | 5.007E-01 | 5.894 | 0.997252 |
| 2 | 50 | 0.505215 | 1.50E-02 | 5.037E-01 | 29.433 | 1.000000 |
| 3 | 100 | 1.01043 | 3.10E-02 | 5.077E-01 | 58.83 | 1.000000 |

Table 12: Stress and Deformation of Aluminum on Depth Variation

| Depth |  |  |
| --- | --- | --- |
| 10 |  |  |
| 50 |  |  |
| 100 |  |  |

Table 13: Stress and Deformation of Aluminum on Depth Variation - values

| ALUMINIUM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Depth(m) | Load (Mpa) | Deformation(mm) | Deformation(sigmoid) | Stress (Mpa) | Stress(sigmoid) |
| 1 | 10 | 0.101043 | 8.00E-03 | 5.020E-01 | 5.809 | 0.997009 |
| 2 | 50 | 0.505215 | 4.20E-02 | 5.105E-01 | 29.042 | 1.000000 |
| 3 | 100 | 1.01043 | 8.40E-02 | 5.210E-01 | 58.057 | 1.000000 |

Table 14: Stress and Deformation of PVC on depth variation

| Depth |  |  |
| --- | --- | --- |
| 10 |  |  |
| 50 |  |  |
| 100 |  |  |

Table 15: Stress and Deformation of PVC on depth variation - values

| PVC UNPLASTICIZED |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Depth(m) | Load (Mpa) | Deformation(mm) | Deformation(sigmoid) | Stress (Mpa) | Stress(sigmoid) |
| 1 | 10 | 0.101042 | 0.758 | 6.809E-01 | 5.577 | 0.996230 |
| 2 | 50 | 0.505215 | 3.788 | 9.779E-01 | 27.889 | 1.000000 |
| 3 | 100 | 1.01043 | 7.572 | 9.995E-01 | 55.754 | 1.000000 |

Table 16: Stress and Deformation of Acrylic on depth variation

| Depth |  |  |
| --- | --- | --- |
| 10 |  |  |
| 50 |  |  |
| 100 |  |  |

Table 17: Stress and Deformation of Acrylic on depth variation - values

| ACRYLIC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Depth(m) | Load (Mpa) | Deformation(mm) | Deformation(sigmoid) | Stress (Mpa) | Stress(sigmoid) |
| 1 | 10 | 0.101042 | 2.08E-01 | 5.518E-01 | 5.741 | 0.996799 |
| 2 | 50 | 0.505215 | 1.042 | 7.392E-01 | 28.709 | 1.000000 |
| 3 | 100 | 1.01043 | 2.083 | 8.892E-01 | 57.394 | 1.000000 |

Stress and deformation limitations are calculated to determine the value of stress and deformation permitted to the material installed. Structure stress and deformation that overvalue the allowable limit are not recommended to be used in the design.

Allowable stress can be expressed by:

|  | 10 |
| --- | --- |

Table 18: Properties of materials

| Properties | Steel | Aluminium | PVC | Acrylic |
| --- | --- | --- | --- | --- |
| Density | 7.850E-06 kg / mm^3 | 2.700E-06 kg / mm^3 | 1.300E-06 kg / mm^3 | 1.188E-06 kg / mm^3 |
| Young’s Modulus | 210000.00 MPa | 68900.00 MPa | 6.97 MPa | 2740.00 MPa |
| Poisson’s Ratio | 0.30 | 0.33 | 0.39 | 0.355 |
| Yield Strength | 207.00 MPa | 275.00 MPa | 11.00 MPa | 48.90 MPa |
| Ultimate Tensile Strength | 345.00 MPa | 310.00 MPa | 11.00 MPa | 79.80 MPa |

Table 19: Allowable stress for the different materials

| No | Material | Stress (MPa) |
| --- | --- | --- |
| 1 | Steel | 115 |
| 2 | Aluminium | 77.5 |
| 3 | PVC | 18.75 |
| 4 | Acrylic | 10.4 |

Figure 12: Deformation comparison of each material at different depths

Figure 13: Stress comparison of each material at different depths

The analysis shows that the stress value of the ROV with every material variation is within allowable values in all operational conditions. The differences in stress and deformation are caused by the values of yield strength and modulus of elasticity that affect the material. However, aluminium is susceptible to corrosion in marine environment and require treatments such as anodizing which add to the cost and maintenance requirements. Further, aluminium requires techniques for welding and precise machining. Acrylic offers superior UV resistance than PVC preventing degradation from sunlight exposure. It is also lightweight in comparison to PVC and has a polished appearance.

The Sealing Compartment and End Caps

The acrylic tube is a cylinder of 400mm length and an exterior diameter of 110mm. The acrylic tube's inner diameter is about 105mm as is seen in the construction of Miniature ROVs [29].

Figure 14: Modelled acrylic sealing compartment

The sealing compartment is subjected to a stress and deformation analysis and the results are tabulated in table 20 and 21 below.

Table 20: Stress and Deformation of Acrylic sealing compartment at 100m depth.

Table 21: Stress and Deformation of Acrylic on depth variation - values

| No | Depth(m) | Load (Mpa) | Deformation(mm) | Stress (Mpa) |
| --- | --- | --- | --- | --- |
| 1 | 100 | 1.010430 | 7.955E-08 | 7.146E-05 |

Figure 18: End Caps

It is observed that the acrylic cylindrical sealing compartment holds under the pressure experienced at a depth of 100m.

## 4.3	Determining the weight and displacement of the ROV.

The information on the weight of the ROV is vital for designing the buoyancy system and ensuring that the ROV operates effectively under water. Table 22 below shows the ROV components and their corresponding mass. It also depicts the total weight of the ROV.

Table 22: Total Mass of the ROV

| Item | Qty | Mass [ Kg] | Size (mm) | Total mass [ Kg] |
| --- | --- | --- | --- | --- |
| Frame |  |  |  |  |
| Acrylic Plexiglass | 1 | 0.45 | 400 x D110 | 0.45 |
| O-ring seals | 6 | 0.086 | - | 0.516 |
| Frame | 1 | 13.39325 | 550 x 550 x 210.11 | 13.39325 |
| Electrical |  |  |  |  |
| Battery for motors | 6 | 0.2 | 94 x 94 | 1.2 |
| Battery for electrical components | 1 | 0.2 |  | 0.2 |
| Raspberry Pi 3 | 1 | 0.1 | 8.5× 5.6× 2 | 0.1 |
| Heatsinks for raspberry Pi 3 | 3 | 0.05 | 14 x 14 x 6 | 0.15 |
| Relays for motors | 6 | 0.015 | 70 × 50 × 20 | 0.09 |
| ESC30A | 6 | 0.03 | 139 x 28 | 0.18 |
| Brushless Motor | 6 | 0.06 | 50 x 30 x 30 | 0.36 |
| Camera | 1 | 0.063 | 70 x 50.8 x 43.9 | 0.063 |
| LED lights | 2 | 0.05 |  | 0.1 |
| Led driver | 2 | 0.004 | 25.3 x 25.3 x 10.3 | 0.08 |
| waterproof LED Lens | 2 | 0.025 |  | 0.05 |
| SD card | 1 | 0.002 | 32.0 × 24.0 × 2.1 | 0.002 |
| USB-type A connector | 1 | 0.04 |  | 0.04 |
| MOSFET IRF1404 | 1 | 0.02 | ‎5 x 2.76 x 0.31 | 0.02 |
| Pressure sensor | 1 | 0.02 |  | 0.02 |
| Nylon plastic | 1 | 0.5 |  | 0.5 |
| Pixhawk controller | 1 | 0.05 |  | 0.05 |
| Motor driver | 2 | 0.06 | 56 x 57 | 0.12 |
| Pulse width modulation | 6 | 0.09 | 100 × 80 × 40 | 0.54 |
| Propulsion |  |  |  |  |
| Thrusters | 6 | 0.075 | ‎72 x 92 | 0.45 |
|  |  | Total Mass |  | 18.67425 |
|  |  | Total weight |  | 183.1943925 |

From table 22, the estimated mass of the ROV is 18.67425kg, estimated weight, taking gravitational pull as 9.81m/s2, is 183.1943925N, and the volume of displacement is 1.127E+7mm3.

## 4.4 Calculating the buoyant force, hydrodynamic drag force, power of the motor, and the thrust Force.

The factors considered in the selection of a suitable motor for the ROV to achieve a designed speed of 2m/s include the hydrodynamic drag, thrust required and power needed.

### 4.4.1	ROV Upthrust Force

To find the force of upthrust experienced by the ROV, equation [2] is used.

### 4.4.2	ROV buoyancy

To find the force of buoyancy experienced by the ROV, equation [3] is used

| Fb= 8.0E-6 kg/m3 x 9.81 x 0.01127 Fb=8.84e-7N |  |
| --- | --- |

### 4.4.3 ROV hydrodynamic drag

The thrust required to achieve a speed of 0.5m/s depends on the hydrodynamic drag force acting on the ROV. The drag force on an ROV moving through water can be calculated using the drag equation [4]

The Reynolds number is used to predict the flow patterns in fluid dynamics. It is a relation between density ρ, characteristic length L, velocity, U and fluid viscosity μ. The Reynolds number for the ROV in the water with ρ = 1025 kg/m³ and μ = 1.31 Cp =1.31 x 10³ kg/ms and moving at 0.5 m/s is calculated below.

The Reynolds number exceed 4000 signifying the flow to be turbulent. For turbulent flow around a smooth sphere, the coefficient of drag levels off to an approximately constant value as the Reynolds number increases, particularly beyond Re≈105.

The force of drag on the ROV vehicle is thus calculated by taking the values below:

Designed ROV speed = 0.5m/s

Density of sea water =1025kg/m3

Wetted surface area of the ROV= 1.763m2

Coefficient of drag = 0.03

The total drag of the ROV system is the vehicle drag, and the tether drag. The coefficient of drag for tether cables used in Remotely Operated Vehicle (ROV) applications vary depending on the cable's surface characteristics, diameter, and flow conditions. The tether drag coefficient in a turbulent flow regime is approximated to be 1.2 [31].  With a tether cable of 0.01m and a length of 10m, the characteristic area of the tether cable is obtained as 0.004712388m3. The drag force on the tether cable is thus calculated in the equation [5].

The total hydrodynamic drag force on the ROV system is thus the summation of the tether drag, and the vehicle drag, and it is given by equation [6]

| FD(Total) = 7.245+6.776=14.02N |  |
| --- | --- |

Power of motor analysis

To calculate the power of a motor needed to achieve a maximum speed of 1m/s and overcome a drag force of 14.02N, the fundamental relationship between power, force and velocity is applied in equation [7]

By including 20% losses, the maximum power of the motor is obtained from equation (Caption).

| W |  |
| --- | --- |

The motors need to produce 16.824W, whish is equivalent to 0.022555992hp to maintain a maximum speed of 1m/s while overcoming the ROV system drag force of 14.02N. It is assumed that the drag force remains constant at the designed speed.

## 4.5 The electrical design

Figure 15 below depicts the main bread board circuitry. Figure 16 shows the circuit for the motor and motor divers. Figure 17 indicates a circuit for sensors and modules.

Figure 15: Schematic circuit Main Board

Figure 16: Schematic circuit Motor and motor drivers

Figure 17: Schematic circuit sensors and modules

# Conclusion

There are challenges experienced in hull inspections of marine vessels. These challenges include costly and time-consuming dry-docking operations, as well as safety risks exposed to the underwater inspectors. The use of ROVs for underwater inspection exercises is deemed a potential solution to these challenges. This report proposes specific objectives which include the design, fabrication and testing of a control system of an underwater ROV. This being the first phase of this undertaking, it is proposed that the ROV design be based on a shallow marine vessel with an average draught of 10 m and therefore a design depth of 10 m is adopted for the ROV in this report. There are two categories of design in this work, namely mechanical, and electssssrical design works. At the end of the project, it is expected that a control system of the ROV with manoeuvrable controls along the main axes of motion as well as around an obstacle underwater will be realized.

# 6.Budget

Table 23: Budget

| ; | NUMBER OF ITEMS | COST PER ITEM (KSH) | TOTAL COST (KSH) |
| --- | --- | --- | --- |
| Brushless motors | 3 | 1500 | 4,500 |
| propellor | 3 | 500 | 1,500 |
| Battery 9v | 2 | 780 | 1,560 |
| Raspberry Pico | 1 | 1500 | 1,500 |
| L239D driver | 3 | 200 | 600 |
| Cables |  | 500 | 500 |
| Relays | 3 | 60 | 180 |
| Breadboard | 1 | 400 | 400 |
|  |  | Total | 10,740 |

# 7. Time plan

Table 24: Time Plan

|  | May | Jun | Jul | Aug | Sept | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Literature Review |  |  |  |  |  |  |  |  |
| Main structural design |  |  |  |  |  |  |  |  |
| Electronic design |  |  |  |  |  |  |  |  |
| Raspberry implementation |  |  |  |  |  |  |  |  |
| Assembly |  |  |  |  |  |  |  |  |
| Testing |  |  |  |  |  |  |  |  |
| Final Presentation |  |  |  |  |  |  |  |  |

# References

[1]	Alpha Logistics Services, "https://www.alphalogisticsafrica.com/download/TUGS/Aylah%20-%201%20v1.0.pdf," 2013.

[2]	M. a. C. Agency, "In-water Surveys," Marine Guidance Note - MGN 546(M). .

[3]	B. Lin and X. Dong, " "Ship hull inspection: A survey," Ocean Engineering, vol. 289, p. 116281, 2023/12/01/ 2023, doi: ."

[4]	B. Lin and X. Dong, ""Ship Hull Inspection: A Survey," Ocean Engineering, ," 11/05 2023.

[5]	N. Lahkar and S. Gautam, ""Applications of Remotely Operated Vehicles (ROVs) in Offshore Operations: A Review on Recent Developments," " 09/15 2023.

[6]	H. PARSONS, ""An insight into the history of ROVs." ".

[7]	B. Jacques, ""Hempel launches underwater hull inspections using ROVs," S. Maritime, Ed., ed,," Sept 2020.

[8]	O. Y. Y. Ahmed, and B. Sun, , ""Design of a new low cost ROV vehicle," Jurnal Teknologi, vol. 69, 07/20 2014, doi: 10.11113/jt. v69.3262.."

[9]	P. I. Macreadie et al., Eyes in the sea: unlocking the mysteries of the ocean using industrial, remotely operated vehicles (ROVs)," Science of the Total Environment, vol. 634, pp. 1077-1091, . 2018.

[10]	NOAA, ""Remotely Operated Vehicle," Ocean Engineering."

[11]	A. Wylie, ""ROV Ventana completes 4500 Dives Deep Sea Dives." ".

[12]	D. J. I. Hudson, and D. Wigham, " A review of the uses of work-class ROVs for the benefits of science: lessons learned from the SERPENT project," Underwater Technology, vol. 26, no. 3, pp. 83-88, . 2005.

[13]	G. P. J. Lever, and J. Weale, , "Benefits of Using Remotely Operated Vehicles to Inspect USACE Navigation Structures," p. 28, 03/01 2007.

[14]	E. Allmendinger, "Submersible vehicle systems de- sign. The society of naval architects and marine engineers, Jersey City (NJ)." 1990.

[15]	J. Smith, " Principles of Marine Engineering. New York: Maritime Press.," (2010).

[16]	A. O. M. D. Patryk Zając, ""Stability Testing Method for Underwater ROVs, ," no. Polish  Hyperbaric Research 1(74)2021,," 2021.

[17]	T. Wilson, " Underwater Robotics: Principles and Practices. Cambridge: Marine Tech Publishers.," (2018).

[18]	M. Jones, "Advances in Underwater Robotics. San Francisco: Oceanic Research Publications.," (2017).

[19]	R. Davis, " Materials in Marine Engineering. London: Oceanic Materials Publishing.," (2016).

[20]	S. Miller, " Innovations in Underwater Vehicle Design. New York: Marine Engineering Press.," (2019).

[21]	P. Anderson, "Materials for Marine Applications. Boston: Coastal Engineering Publications.," (2020).

[22]	J. Taylor, "Dynamics of Underwater Vehicles. Houston: Marine Systems Press.," (2021).

[23]	A. Walker, "Hydrodynamics and Marine Propulsion. San Diego: Oceanic Engineering Publications.," (2018).

[24]	D. Harris, "Marine Propulsion Systems and Design. Seattle: Nautical Engineereing Press," (2019).

[25]	L. Martin, " Underwater Vehicle Propulsion and Dynamics. Miami: Ocean Engineering Publications.," (2020).

[26]	T. Nguyen, "Advanced Marine Propulsion and Hydrodynamics. Cambridge: ," (2021).

[27]	R. Johnson, "Fluid Dynamics of Underwater Tethers. Oxford," (2019).

[28]	P. Clark, " Hydrodynamics of Autonomous Underwater Vehicles," (2022).

[29]	J. Smith, Brown, T., & Wilson, L., "Design and Construction of Miniature ROVs. Journal of Marine Engineering and Technology, 22(3), 45-58.," (2018).

[30]	B. R. Munson, Young, D. F., Okiishi, T. H., & Huebsch, W. W., "Fundamentals of Fluid Mechanics (7th ed.). Hoboken, NJ: Wiley.," (2013).

[31]	A. Stevens, "Hydrodynamics of Underwater Tether Systems," (2017).

## Appendix

Motor Control implementation

# Code to drive and change the speed of the motors # !/usr/bin/env python3 # Modules from goto import with_goto from stddef import * import var import pio import resource from datetime import datetime import RPi.GPIO as GPIO import time import cpu import FileStore import VFP def peripheral_setup () : # Peripheral Constructors pio.cpu=cpu.CPU () pio.storage=FileStore.FileStore () pio.server=VFP.VfpServer () pio.storage.begin () pio.server.begin (0) # Install interrupt handlers def peripheral_loop () : pio.server.poll () class MotorController: def __init__(self, pwm_pin, in1_pin, in2_pin, in3_pin=None, in4_pin=None, in5_pin=None, in6_pin=None): self.pwm_pin = pwm_pin self.in1_pin = in1_pin self.in2_pin = in2_pin self.in3_pin = in3_pin self.in4_pin = in4_pin self.in5_pin = in5_pin self.in6_pin = in6_pin GPIO.setup(self.pwm_pin, GPIO.OUT) if self.in1_pin is not None: GPIO.setup(self.in1_pin, GPIO.OUT) if self.in2_pin is not None: GPIO.setup(self.in2_pin, GPIO.OUT) if self.in3_pin is not None: GPIO.setup(self.in3_pin, GPIO.OUT) if self.in4_pin is not None: GPIO.setup(self.in4_pin, GPIO.OUT) if self.in5_pin is not None: GPIO.setup(self.in5_pin, GPIO.OUT) if self.in6_pin is not None: GPIO.setup(self.in6_pin, GPIO.OUT) self.pwm = GPIO.PWM(self.pwm_pin, 100) self.pwm.start(0) def move(self, speed=100): if self.in1_pin is not None: GPIO.output(self.in1_pin, GPIO.HIGH) if self.in2_pin is not None: GPIO.output(self.in2_pin, GPIO.LOW) if self.in3_pin is not None: GPIO.output(self.in3_pin, GPIO.HIGH) if self.in4_pin is not None: GPIO.output(self.in4_pin, GPIO.LOW) if self.in5_pin is not None: GPIO.output(self.in5_pin, GPIO.HIGH) if self.in6_pin is not None: GPIO.output(self.in6_pin, GPIO.LOW) self.pwm.ChangeDutyCycle(speed) def stop(self): self.pwm.stop() GPIO.cleanup() class MotorSystem: def __init__(self): self.ZN_AXIS_MOTORS = MotorController(26, 22, 23, 25, 24) self.YN_AXIS_MOTORS = MotorController(21, 18, 27, 4, 17) self.XN_AXIS_MOTORS = MotorController(21, 27, 18) self.ZP_AXIS_MOTORS = MotorController(26, 23, 22, 24, 25) self.YP_AXIS_MOTORS = MotorController(21, 27, 18, 17, 4) self.XP_AXIS_MOTORS = MotorController(21, 17, 4) self.UP_DIRECTION_PIN = 6 # GO UP self.DOWN_DIRECTION_PIN = 5 # GO DOWN self.LEFT_DIRECTION_PIN = 20 # GO LEFT self.RIGHT_DIRECTION_PIN = 19 # GO RIGHT self.FORWARD_DIRECTION_PIN = 16 # GO FORWARD self.BACK_DIRECTION_PIN = 13 # REVERSE GPIO.setup(self.UP_DIRECTION_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) GPIO.setup(self.DOWN_DIRECTION_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) GPIO.setup(self.LEFT_DIRECTION_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) GPIO.setup(self.RIGHT_DIRECTION_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) GPIO.setup(self.FORWARD_DIRECTION_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) GPIO.setup(self.BACK_DIRECTION_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) GPIO.add_event_detect(self.UP_DIRECTION_PIN, GPIO.BOTH, callback=self.UP_DIRECTION , bouncetime=200) GPIO.add_event_detect(self.DOWN_DIRECTION_PIN, GPIO.BOTH, callback=self.DOWN_DIRECTION, bouncetime=200) GPIO.add_event_detect(self.LEFT_DIRECTION_PIN, GPIO.BOTH, callback=self.LEFT_DIRECTION, bouncetime=200) GPIO.add_event_detect(self.RIGHT_DIRECTION_PIN, GPIO.BOTH, callback=self.RIGHT_DIRECTION , bouncetime=200) GPIO.add_event_detect(self.FORWARD_DIRECTION_PIN, GPIO.BOTH, callback=self.FORWARD_DIRECTION, bouncetime=200) GPIO.add_event_detect(self.BACK_DIRECTION_PIN, GPIO.BOTH, callback=self.BACK_DIRECTION, bouncetime=200) def UP_DIRECTION(self): if GPIO.input(self.UP_DIRECTION_PIN) == GPIO.LOW: self.ZP_AXIS_MOTORS.move() else: self.ZP_AXIS_MOTORS.stop() def DOWN_DIRECTION(self): if GPIO.input(self.DOWN_DIRECTION_PIN) == GPIO.LOW: self.ZN_AXIS_MOTORS.move() else: self.ZN_AXIS_MOTORS.stop() def LEFT_DIRECTION(self): if GPIO.input(self.LEFT_DIRECTION_PIN) == GPIO.LOW: self.XN_AXIS_MOTORS.move() else: self.XN_AXIS_MOTORS.stop() def RIGHT_DIRECTION(self): if GPIO.input(self.RIGHT_DIRECTION_PIN) == GPIO.LOW: self.XP_AXIS_MOTORS.move() else: self.XP_AXIS_MOTORS.stop() def FORWARD_DIRECTION(self): if GPIO.input(self.FORWARD_DIRECTION_PIN) == GPIO.LOW: self.YN_AXIS_MOTORS.move() else: self.YN_AXIS_MOTORS.stop() def BACK_DIRECTION(self): if GPIO.input(self.BACK_DIRECTION_PIN) == GPIO.LOW: self.YP_AXIS_MOTORS.move() else: self.YP_AXIS_MOTORS.stop() def main(): print("Press the button to start the motor.") # SETUP peripheral_setup() motor_system = MotorSystem() try: while True: time.sleep(1) # Wait for button press except KeyboardInterrupt: pass finally: motor_system.ZN_AXIS_MOTORS.stop() motor_system.XN_AXIS_MOTORS.stop() motor_system.YN_AXIS_MOTORS.stop() motor_system.ZP_AXIS_MOTORS.stop() motor_system.XP_AXIS_MOTORS.stop() motor_system.YP_AXIS_MOTORS.stop() # Command line execution if __name__ == '__main__' : main()

Sensors and feedback modules code implementation

#include <Wire.h> #include <MPU6050.h> #define kpa2atm 0.00986923267 const int Pin = 24; const int pressurePin = 23; int val; float pkPa; // pressure in kPa float pAtm; // pressure in Atm unsigned long time; MPU6050 mpu; void setup() { Serial.begin(9600); Wire.begin(); mpu.initialize(); if (!mpu.testConnection()) { Serial.println("MPU6050 connection failed"); while (1); } Serial.println("MPU6050 connection successful"); } void loop() { // Get the temperature int vout = analogRead(Pin); float tempc = (vout * 4.88) / 10; // Get the pressure val = analogRead(pressurePin); pkPa = ((float)val / (float)1023 + 0.095) / 0.009; pAtm = kpa2atm * pkPa; // Get IMU readings mpu.getMotion6(&ax, &ay, &az, &gx, &gy, &gz); // Send to serial port Serial.print("Temperature: "); Serial.print(tempc); Serial.println(" deg Celsius"); Serial.print("Pressure: "); Serial.print(pkPa); Serial.print(" kPa "); Serial.print(pAtm); Serial.println(" Atm"); Serial.print("Acceleration: "); Serial.print("X = "); Serial.print(ax); Serial.print(" ¦ Y = "); Serial.print(ay); Serial.print(" ¦ Z = "); Serial.println(az); Serial.print("Gyroscope: "); Serial.print("X = "); Serial.print(gx); Serial.print(" ¦ Y = "); Serial.print(gy); Serial.print(" ¦ Z = "); Serial.println(gz); delay(1000); }